# A Quick User Manual

# SignalBrowser – A psychophysiology Signal Processing and Analysis Toolbox

SignalBrowser is a graphic user interface software tool written in Matlab to load, process and analyze psychophysiological data (ECG, respiration and skin conductance). This software can do the following:

- Filter and process ECG, respiration (RESP), and skin conductance (SCL)

- Automatically detect and subsequently manually edit ECG R-peaks to compute the RR or interbeat-interval (IBI) signal

- Compute ECG derived respiratory signal (EDR)

- Compute several Heart Rate Variability (HRV) measures of IBI in the time (Mean RR, RMSSD…) and frequency (ultra low, low and high relative and absolute frequency power) domains.

- Compute several frequency domain measures of RESP and EDR

- Compute Galvanic skin conductance level (SCL) or responses (SCR)

- Compute the time frequency power spectrum using wavelet (Morlet, Dog, Paul) or Fourier Transforms (second screenshots on next page, bottom plot)

- Compute the time course of the low (LF), high (HF) and low to high ratio (LF/HF) of the HRV frequency spectrum and related frequency of the peak respiratory sinus arrhythmia (RSA; frequency of the high frequency HRV power spectrum) and corresponding respiratory signal (RESP, EDR) as a measure of the sympathetic (LF) and parasympathetic (HF) balance of the autonomic nervous system.

- HRV or RESP/EDR power spectrum analyses can be performed using various computational methods (Fourier transform, Wavelet, Welch, Welch-Spargle, Autoregressive based power spectrum density calculation) and several conditioning window filtering (Hamming, Hanning, Square, Barlett, Blackman…)

- Export outcome measures in a file for each of the defined analysis “Sample” data segments

The RR editing view allows you to principally load ECG and other signals and compute/edit ECG R-peaks to extract the RR or interbeat-interval signal (IBI). It can also allow you to define

- Some “Sample” data segments (manually or interactively with a click and drag of the mouse) from which outcome measures are computed

- Some noisy “Signal” data segments (manually or with the mouse) which are excluded from the computation of outcome measures.

The All Signal view allows you to scroll through and visualize all signals (IBI, RESP, EDR, SCL), as well as outcome measures for each of the “Sample” data segments. It also computes and displays the time-frequency power spectrum of IBI and the time course of the autonomic nervous system (ANS) represented by the low, high and low to high ratio of the HRV frequency spectrum and related time course of the frequency of the peak respiratory sinus arrhythmia (RSA; frequency of the high frequency HRV power spectrum) and corresponding respiratory signal (RESP, EDR)

Zoom in (red) window for R-peaks editing

RR Editing View – Sample Editing View

Toggle between RR editing/All signals view

Display “All” or selectively SCL (skin conductance), RESP (respiration), EDR (ECG derived respiration), IBI (interbeat intervals) data

Zoom in to the selected Sample segment. Scroll left (<<) or right (>>) or delete a given Sample data segments. Analysis metrics on the right are computed on the selected Sample segment

Automatically find R peaks

Zoom out (double the range of the time and IBI axis limit at every click)

Compute ECG derived respiration (using amplitude modulation of R peaks)

Invert ECG if necessary

Scroll left/right the zoom in R-peaks editing window

Automatic and manual R-peaks editing window

Signal viewing for creating and editing “Signal” and “Sample” data segments. “Signal” magenta shaded regions define noisy data segment.

“Sample” green shaded regions define analysis data samples segments.

Full time view of ECG data

HRV analysis option editing

Analysis Sample data segment selection and characteristics

Data Measures (HRV, SCL level and response, Respiration freq.) for the selected Analysis Sample window

Freq spectrum for HRV, Respiration and EDR for the selected Analysis Sample window

Sample windows (green) defining data segments to be analyzed

Toggle between Signal and Sample editing view and editing

Zoom in (red) window for R-peaks editing

Signal window (magenta) defining noisy data segments

RR Editing View – Signal Editing View

Zoom in window for R-peaks editing

Toggle between RR editing view and All signals view

Display “All” or selectively SCL (skin conductance), RESP (respiration), EDR (ECG derived respiration), IBI (interbeat intervals) data

Zoom in to the selected Signal segment window. Scroll left (<<) or right (>>) or delete a given Signal data segment.

Automatically find R peaks

Zoom out (double the range of the time and IBI axis limit at every click)

Compute ECG derived respiration (using amplitude modulation of R peaks)

Invert ECG if necessary

Scroll left/right the R-peaks zoom in window

R-peaks automatic and manual editing window

Signal viewing for creating and editing “Signal” and “Sample” data segments. “Signal” magenta shaded regions define noisy data segments.

“Sample” green shaded regions define analysis data segments.

Full time view of ECG data

HRV analysis option editing

Editing IBI “Signal” segment selection and characteristics

Data Measures (HRV, SCL level and response, Respiration freq.) for the selected Analysis Sample window

Freq spectrum for HRV, Respiration and EDR for the selected Analysis Sample window

Signal window (magenta) defining noisy data segments

Zoom in window for R-peaks editing

Toggle between Signal and Sample editing view and editing

All Signals View

Toggle between HRV Power and time frequency Spectrum view

HRV Power (top) or time frequency Spectrum view (bottom)

Signal viewing showing SCL/ RESP & EDR / IBI signal data in separate plots

Zoom in to the selected Sample segment. Scroll left (<<) or right (>>) or delete a given Sample data segments. Analysis metrics on the right are computed on the selected Sample segment

Toggle between Signal and Sample editing view and editing

Zoom out (double the range of the time and IBI axis limit at every click)

HRV analysis option editing

Editing IBI “Signal” segment selection and characteristics

Data Measures (HRV, SCL level and response, Respiration freq.) for the selected Analysis Sample window

Freq spectrum for HRV, Respiration and EDR for the selected Analysis Sample window

Select Spectrum Method